SUPPORTING INFORMATION

Parametrization of biowaste composting system for Life Cycle Assessment

Nomena Ravoahangy, Guillaume Majeau-Bettez, Olivier Schoefs

Table of contents

Note S1.	Composting model details	2

Note S2.	Details of the parametrization	19

Note S3.	Initial values for the simulation	20

Note S4.	Inventory data for LCA simulations	20

Note S5.	Validation of active composting model	22

Note S6.	LCA results	23

# Composting model details

All the reactions of the composting model are detailed below using Pertersen matrix.

- Methodology for model development

- Hydrolysis phase

| n | C | P | L | H | CE | LG | Sc | Sp | Sl | Sh | Slg | Reaction rate (kg/kgTM.h) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | -1 |  |  |  |  |  | 1 |  |  |  |  | v1=kh1CCkhS.XMB+CXMB |
| 2 |  | -1 |  |  |  |  |  | 1 |  |  |  | v2=kh2PPkhS.XMB+PXMB |
| 3 |  |  | -1 |  |  |  |  |  | 1 |  |  | v3=kh3LLkhS.XMB+LXMB |
| 4 | -1 |  |  |  |  |  | 1 |  |  |  |  | v4=kh4CCkhS.XTB+CXTB |
| 5 |  | -1 |  |  |  |  |  | 1 |  |  |  | v5=kh5PPkhS.XTB+PXTB |
| 6 |  |  | -1 |  |  |  |  |  | 1 |  |  | v6=kh6LLkhS.XTB+LXTB |
| 7 |  |  |  | -1 |  |  |  |  |  | 1 |  | v7=kh7HHkhS.XTA+HXTA |
| 8 |  |  |  |  | -1 |  | 1 |  |  |  |  | v8=kh8CECEkhS.XTF+CEXTF |
| 9 |  |  |  |  |  | -1 |  |  |  |  | 1 | v9=kh9LGLGkhS.XTF+LGXTF |
| 10 |  |  |  | -1 |  |  |  |  |  | 1 |  | v10=kh10HHkhS.XMA+HXMA |
| 11 |  |  |  |  | -1 |  | 1 |  |  |  |  | v11=kh11CECEkhS.XMF+CEXMF |
| 12 |  |  |  |  |  | -1 |  |  |  |  | 1 | v12=kh12LGLGkhS.XMF+LGXMF |

- Growth of heterotrophic microorganisms on soluble substrate

|  | Sc | Sp | Sl | Sh | Slg | O2 | Xmb | Xtb | Xma | Xta | Xmf | Xtf | CO2 | H2O | NH4 | Reaction rate (kg/kgTM.h) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13 | -1Yx/s |  |  |  |  | -1Y13,x/O2 | 1 |  |  |  |  |  | 1Y13,x/CO2 | 1Y13,x/H2O | -1Y13,x/N | v13=µmb.fSc.fT.fO2.XMB |
| 14 |  | -1Yx/s |  |  |  | -1Y14,x/02 | 1 |  |  |  |  |  | 1Y14,x/C02 | 1Y14,x/H2O | 1Y14,x/N | v14=µmb.fSp.fT.fO2.XMB |
| 15 |  |  | -1Yx/s |  |  | -1Y15,x/O2 | 1 |  |  |  |  |  | 1Y15,x/CO2 | 1Y15,x/H2O | -1Y15,x/N | v15=µmb.fSl.fT.fO2.XMB |
| 16 | -1Yx/s |  |  |  |  | -1Y16,x/O2 |  | 1 |  |  |  |  | 1Y16,x/CO2 | 1Y16,x/H2O | -1Y16,x/N | v16=µTB.fSc.fT.fO2.XTB |
| 17 |  | -1Yx/s |  |  |  | -1Y17,x/02 |  | 1 |  |  |  |  | 1Y17,x/C02 | 1Y17,x/H2O | 1Y17,x/N | v17=µTB.fSp.fT.fO2.XTB |
| 18 |  |  | -1Yx/s |  |  | -1Y18,x/O2 |  | 1 |  |  |  |  | 1Y18,x/CO2 | 1Y18,x/H2O | -1Y18,x/N | v18=µTB.fSl.fT.fO2.XTB |
| 19 | -1Yx/s |  |  |  |  | -1Y19,x/O2 |  |  | 1 |  |  |  | 1Y19,x/CO2 | 1Y19,x/H2O | -1Y19,x/N | v19=µMA.fSc.fT.fO2.XMA |
| 20 |  | -1Yx/s |  |  |  | -1Y20,x/02 |  |  | 1 |  |  |  | 1Y20,x/C02 | 1Y20,x/H2O | 1Y20,x/N | v20=µMA.fSp.fT.fO2.XMA |
| 21 |  |  | -1Yx/s |  |  | -1Y21,x/O2 |  |  | 1 |  |  |  | 1Y21,x/CO2 | 1Y21,x/H2O | -1Y21,x/N | v21=µMA.fSl.fT.fO2.XMA |
| 22 |  |  |  | -1Yx/s |  | -1Y22,x/O2 |  |  | 1 |  |  |  | 1Y22,x/CO2 | 1Y22,x/H2O | -1Y22,x/N | v22=µMA.fSh.fT.fO2.XMA |
| 23 | -1Yx/s |  |  |  |  | -1Y23,x/O2 |  |  |  | 1 |  |  | 1Y23,x/CO2 | 1Y23,x/H2O | -1Y23,x/N | v23=µTA.fSc.fT.fO2.XTA |
| 24 |  | -1Yx/s |  |  |  | -1Y24,x/02 |  |  |  | 1 |  |  | 1Y24,x/C02 | 1Y24,x/H2O | 1Y24,x/N | v24=µTA.fSp.fT.fO2.XTA |
| 25 |  |  | -1Yx/s |  |  | -1Y25,x/O2 |  |  |  | 1 |  |  | 1Y25,x/CO2 | 1Y25,x/H2O | -1Y25,x/N | v25=µTA.fSl.fT.fO2.XTA |
| 26 |  |  |  | -1Yx/s |  | -1Y26,x/O2 |  |  |  | 1 |  |  | 1Y26,x/CO2 | 1Y26,x/H2O | -1Y26,x/N | v26=µTA.fSh.fT.fO2.XTA |
| 27 | -1Yx/s |  |  |  |  | -1Y27,x/O2 |  |  |  |  | 1 |  | 1Y27,x/CO2 | 1Y27,x/H2O | -1Y27,x/N | v27=µMF.fSc.fT.fO2.XMF |
| 28 |  | -1Yx/s |  |  |  | -1Y28,x/02 |  |  |  |  | 1 |  | 1Y28,x/C02 | 1Y28,x/H2O | 1Y28,x/N | v28=µMF.fSp.fT.fO2.XMF |
| 29 |  |  | -1Yx/s |  |  | -1Y29,x/O2 |  |  |  |  | 1 |  | 1Y29,x/CO2 | 1Y29,x/H2O | -1Y29,x/N | v29=µMF.fSl.fT.fO2.XMF |
| 30 |  |  |  | -1Yx/s |  | -1Y30,x/O2 |  |  |  |  | 1 |  | 1Y30,x/CO2 | 1Y30,x/H2O | -1Y30,x/N | v30=µMF.fSh.fT.fO2.XMF |
| 31 |  |  |  |  | -1Yx/s | -1Y31,x/O2 |  |  |  |  | 1 |  | 1Y31,x/CO2 | 1Y31,x/H2O | -1Y31,x/N | v31=µMF.fSlg.fT.fO2.XMF |
| 32 | -1Yx/s |  |  |  |  | -1Y32,x/O2 |  |  |  |  |  | 1 | 1Y32,x/CO2 | 1Y32,x/H2O | -1Y32,x/N | v32=µTF.fSc.fT.fO2.XTF |
| 33 |  | -1Yx/s |  |  |  | -1Y33,x/02 |  |  |  |  |  | 1 | 1Y33,x/C02 | 1Y33,x/H2O | 1Y33,x/N | v33=µTF.fSp.fT.fO2.XTF |
| 34 |  |  | -1Yx/s |  |  | -1Y34,x/O2 |  |  |  |  |  | 1 | 1Y34,x/CO2 | 1Y34,x/H2O | -1Y34,x/N | v34=µTF.fSl.fT.fO2.XTF |
| 35 |  |  |  | -1Yx/s |  | -1Y35,x/O2 |  |  |  |  |  | 1 | 1Y35,x/CO2 | 1Y35,x/H2O | -1Y35,x/N | v35=µTF.fSh.fT.fO2.XTF |
| 36 |  |  |  |  | -1Yx/s | -1Y36,x/O2 |  |  |  |  |  | 1 | 1Y36,x/CO2 | 1Y36,x/H2O | -1Y36,x/N | v36=µMF.fSlg.fT.fO2.XMF |

- Death and lysis of heterotroph microorganisms

|  | Xmb | Xtb | Xma | Xta | Xmf | Xtf | Xdb | Xi | P | Reaction rate (kg/kgTM.h) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 37 | -1 |  |  |  |  |  | 1 |  |  | v37=bMB.XMB |
| 38 |  | -1 |  |  |  |  | 1 |  |  | v38=bTB.XTB |
| 39 |  |  | -1 |  |  |  | 1 |  |  | v39=bMA.XMA |
| 40 |  |  |  | -1 |  |  | 1 |  |  | v40=bTA.XTA |
| 41 |  |  |  |  | -1 |  | 1 |  |  | v41=bMF.XMF |
| 42 |  |  |  |  |  | -1 | 1 |  |  | v42=bTF.XTF |
| 43 |  |  |  |  |  |  | -1 | fi | (1-fi) | v43=kdec.XDB |

- Nitrogen cycle

|  | NH4 | O2 | Xa | H2O | NO3 | N2O | N2 | Xi | P | NH3gas | NH3 | Reaction rate (kg/kgTM.h) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 44 | -1YXaNH4 | -1YXaO2 | 1 | 1YXaH2O | 1YXaNO3 |  |  |  |  |  |  | v44=µa.fNH4,nit.fO2,nitXa |
| 45 |  |  |  |  | -1 | YN2ONO3 | YN2NO3 |  |  |  |  | v45=pmaxdenit.NO3 .flimNO3.flimTdenit |
| 46 |  |  | -1 |  |  |  |  | fi | (1-fi) |  |  | v46=ba.Xa |
| 47 | -1 |  |  |  |  |  |  |  |  | 1 |  | v47a=klaNH4.(NH4-NH4,eq) klaNH4=100.exp⁡(0.03.T-298) NH4,eq= kH,NH3.PNH3 klaNH4: transfer coefficient (h-1) NH4,eq: quantity of NH4 in the liquid-gas interface (kg/kgTM) PNH3: Pressure of NH3 in the gas phase (Pa) |
|  |  |  |  |  |  |  |  |  |  | -1 | 1 | v47b=Qair.NH3gasVgas Qair: air flow rate (m3.h-1) Vgas: Volume of gas (m3) |

- Methane module

|  | CH4,gen | O2 | CO2 | H2O | Reaction rate |
| --- | --- | --- | --- | --- | --- |
| 48a | 1 |  |  |  | v48a=YCH4/Si.RSi.11+η.O2 |
| 48b | -1 | -YO2CH4 | YCO2CH4 | YH2OCH4 | v48b=vmax.fCH4,gen.fO2,met |

*Adapted from 1,2

RSi= υhydrolysisSi

υhydrolysisSi: hydrolysis rates from which Si is obtained

- Limiting functions

Temperature limiting function for microorganism’s growth on soluble substrate fT=(T-Tmax)(T-Tmin)2(Topt-Tmin)(Topt-TminT-Topt-((Topt-Tmax)(Topt+Tmin-2T)

Oxygen limiting function for microorganism’s growth on soluble substrate fO2=sO2*kO2+sO2* so2*=O2H2O : Concentration of dissolved oxygen

Substrate limiting function for microorganism’s growth on soluble substrate fSi=si*ks+si* si*=iH2O : Concentration of i in the liquid phase

Ammonium limiting function for nitrification fNH4,nit=sNH4*Kn+sNH4* Kn=100.051T-7.158 (kg.l-1) (T = temperature, C) 3 sNH4*=NH4H2O: Concentration of NH4 in the liquid phase

Oxygen limiting function for nitrification fO2,nit=sO2*kO2,nit+sO2*

Limitation of denitrification by NO3- flimNO3=sNO3*kNO3+sNO3* sNO3*=NO3-H2O is the concentration of NO3- in the liquid phase

Limitation of denitrification by temperature flimTdenit=expT-11ln89-9ln2.110, if t<11°CflimTdenit=expT-20ln2.110, if t≥11°C

Limitation of methane oxidation by methane generated fCH4,gen=sCH4genKm+sCH4gen sCH4gen=CH4genH2O is the concentration of CH4 generated in the liquid phase

Limitation of methane oxidation by oxygen fO2,met=sO2*kO2,CH4+sO2*

- Energy balance

dTdt=Qbio-Qcond-Qconvcpwastemwaste The heat generated by biological activities: Qbio=RO2×hbio 4 RO2: consumption rate of oxygen (mol.h-1) hbio: heat released per mole of oxygen consumed (kJ.mol-1) mwaste: mass of waste (kgTM) RO2=RGYO2 5 RG: Total growth rate of aerobic microorganisms (mol.h-1) YO2: Biomass yield on oxygen (mol.mol-1) RG=iRGiMi. mwaste RGi: growth rate of aerobic microorganism i (kg.h-1.kgTM-1) Mi: Molar mass of microorganism i (kg.mol-1) The heat loss through conduction: Qcond=T-Ta.U.A U: Heat transfer coefficient (kJ/m2.K.h) A: Surface area of heat conduction (m2) Ta: Ambient temperature (K) The convective heat loss by air flow: Qconv=Ca.Ta-T.Qair Ca : Specific heat of air (kJ/K.kg)

- Moisture balance

dWdt= vproduction-vevaporation vproduction: sum of biological production rates of W vevaporation: rate of evaporation of water vevaporation =klaH2O.(Ps -Pv) klaH2O: Mass transfer coefficient for H2O (kg/h.Pa) Ps: pressure of water vapour saturated at temperature of gas phase (Pa) Pv: pressure of water vapour in gas phase (Pa) Ps= 10(22.443 - (2795/T) - 1.6798.ln(T)) Pv=nvap.R.TVgas

- Variables

| Symbol | Units | Description | Formula |
| --- | --- | --- | --- |
| C | kg.kg-1 | Carbohydrate | C6H1206 |
| P | kg.kg-1 | Protein | C16H24O5N4 |
| L | kg.kg-1 | Lipid | C25H45O3 |
| HE | kg.kg-1 | Hemicellulose | C10H18O9 |
| CE | kg.kg-1 | Cellulose | (C6H12O6)n |
| LG | kg.kg-1 | Lignin | C20H30O6 |
| Xi | kg.kg-1 | Inert materials |  |
| Sc | kg.kg-1 | Hydrolysis product of C and CE | C6H1206 |
| Sp | kg.kg-1 | Hydrolysis product of P | C16H24O5N4 |
| Sl | kg.kg-1 | Hydrolysis product of L | C25H45O3 |
| Sh | kg.kg-1 | Hydrolysis product of H | C10H18O9 |
| Slg | kg.kg-1 | Hydrolysis product of LG | C20H30O6 |
| Xmb | kg.kg-1 | Mesophilic bacterias | C5H7O2N |
| Xtb | kg.kg-1 | Thermophilic bacterias | C5H7O2N |
| Xma | kg.kg-1 | Mesophilic actynomycetes | C5H7O2N |
| Xta | kg.kg-1 | Thermophilic actynomycetes | C5H7O2N |
| Xmf | kg.kg-1 | Mesophilic fungi | C10H17O6N |
| Xtf | kg.kg-1 | Thermophilic fungi | C10H17O6N |
| Xa | kg.kg-1 | Autotroph microorganisms | C5H7O2N |
| Xdb | kg.kg-1 | Decayed biomass | C5H7O2N |
| H2O | kg.kg-1 | Water |  |
| O2 | kg.kg-1 | Dissolved dioxygen |  |
| CO2 | kg.kg-1 | Emitted carbon dioxide |  |
| NH4 | kg.kg-1 | Ammonium in liquid phase |  |
| N2O | kg.kg-1 | Dinitrogen oxide |  |
| N2 | kg.kg-1 | Dinitrogen |  |
| NH3gas | kg.kg-1 | Ammoniac in gas phase |  |
| NH3 | kg.kg-1 | Emitted ammonia |  |
| NO3 | kg.kg-1 | Nitrate |  |
| CH4,gen | kg.kg-1 | Methane produced |  |
| CH4 | kg.kg-1 | Emitted methane |  |
| T | K | Temperature of the pile |  |

- Parameters

| Parameters | Values | Units | Signification | Source |
| --- | --- | --- | --- | --- |
| kh1C | 0.0293 | h-1 | Mesophilic hydrolysis constant of carbohydrates (Xmb) | this study |
| kh2P | 0.0754 | h-1 | Mesophilic hydrolysis constant of proteins (Xmb) | this study |
| kh3L | 1.45E-03 | h-1 | Mesophilic hydrolysis constant of lipids (Xmb) | this study |
| kh4C | 5.30E-03 | h-1 | Thermophilic hydrolysis constant of carbohydrates (Xtb) | this study |
| kh5P | 0.0754 | h-1 | Thermophilic hydrolysis constant of proteins (Xtb) | this study |
| kh6L | 0.0182 | h-1 | Thermophilic hydrolysis constant of lipids (Xtb) | this study |
| kh7H | 0.009 | h-1 | Thermophilic hydrolysis constant of hemicelluloses (Xta) | this study |
| kh8CE | 0.007 | h-1 | Thermophilic hydrolysis constant of celluloses (Xtf) | this study |
| kh9LG | 0.0068 | h-1 | Thermophilic hydrolysis constant of lignins (Xtf) | this study |
| kh10H | 0.009 | h-1 | Mesophilic hydrolysis constant of hemicelluloses (Xma) | this study |
| kh11CE | 0.007 | h-1 | Mesophilic hydrolysis constant of celluloses (Xmf) | this study |
| kh12LG | 0.0068 | h-1 | Mesophilic hydrolysis constant of lignins (Xmf) | this study |
| µMB | 0.2 | h-1 | Specific growth rate for mesophilic bacteria on Sc, Sp, Sl | 6 |
| µTB | 0.18 | h-1 | Specific growth rate for thermophilic bacteria on Sc, Sp, Sl | 6 |
| µMA | 0.1 | h-1 | Specific growth rate of mesophilic actinomycetes on Sc, Sp, Sl, Sh | 6 |
| µTA | 0.12 | h-1 | Specific growth rate of thermophilic actinomycetes on Sc, Sp, Sl, Sh | 6 |
| µMF | 0.1 | h-1 | Specific growth rate of mesophilic fungi on Sc, Sp, Sl, Sh, Slg | 6 |
| µTF | 0.1 | h-1 | Specific growth rate of thermophilic fungi on Sc, Sp, Sl, Sh, Slg | 6 |
| µa | 0.003 | h-1 | Specific growth rate of autotroph microorganisms | this study |
| khS | 1.00E-04 | kg/kg | Saturation coefficient for Contois kinetics | 6 |
| bMB | 0.012 | h-1 | Death constant for mesophilic bacterias | this study |
| bTB | 0.008 | h-1 | Death constant for thermophilic bacterias | this study |
| bMA | 0.01 | h-1 | Death constant for mesophilic actinomycetes | 6 |
| bTA | 0.015 | h-1 | Death constant for thermophilic actinomycetes | 6 |
| bMF | 0.01 | h-1 | Death constant for mesophilic fungis | 6 |
| bTF | 0.01 | h-1 | Death constant for thermophilic fungis | 6 |
| ba | 0.0083 | h-1 | Death constant for autotroph microorganisms | 7 |
| kdec | 0.0025 | h-1 | Microorganisms decomposition constant | 6 |
| kH,NH3 | 0.0006 | mol/l.Pa | Henry constant for NH3 | 8 |
| pmaxdenit | 0.01 | h-1 | maximal rate for denitrification | 8 |
| Tmin | 273 | K | Minimum temperature for microorganisms growth | 6 |
| Topt | 322 | K | Optimum temperature for microorganisms growth | 6 |
| Tmax | 355 | K | Maximum temperature for microorganisms growth | 6 |
| kO2,het | 7.00E-09 | kg/l | Oxygen saturation constant for heterotrophic activities | 6 |
| ks | 6.20E-05 | kg/l | Substrate saturation constant for Monod kinetics | 6 |
| kO2,nit | 6.00E-07 | kg/l | Monod constant for oxygen for nitrification | 9 |
| kNO3 | 8.40E-02 | kg/l | Half-saturation constant for denitrification | 7 |
| vmax | 5.35E-04 | kg/kg.h | Maximum rate of methane oxidation | 10 |
| kO2,CH4 | 3.30E-02 | mol/l | Michaelis constant for oxygen in methane oxidation | 11 |
| η | 4.00E+05 | l/mol | Sensitivity of methanogenesis to inhibition by oxygen | 5 |
| Km | 7.20E-01 | kg/l | Michaelis constant for methane oxidation | 11 |
| fi | 0.3 | - | Proportion of dead biomass recycled to inert materials | this study |
| cpwaste | 2 | kJ/K.kg | Specific heat of waste | 12 |
| hbio | 440 | kJ/mol | Heat released per mole of oxygen consumed | 4 |
| YO2 | 1.12 | mol/mol | Biomass yield on oxygen | 5 |
| Ca | 1 | kJ/K.kg | Specific heat of air | 13 |
| YXaNH4 | 0.13 | kg/kg | Yield coefficient of autotrophic biomass on NH4 | Calculated by stochiometric coefficients from 9 |
| YXaO2 | 0.041 | kg/kg | Yield coefficient of autotrophic biomass on O2 | Calculated by stochiometric coefficients from 9 |
| YXaH2O | 0.127 | kg/kg | Yield coefficient of autotrophic biomass on H2O | Calculated by stochiometric coefficients from 9 |
| YXaNO3 | 0.039 | kg/kg | Yield coefficient of autotrophic biomass on NO3 | Calculated by stochiometric coefficients from 9 |
| YN2ONO3 | 0.071 | kg/kg | Yield coefficient of N2O on NO3 | this study |
| YN2NO3 | 0.181 | kg/kg | Yield coefficient of N2 on NO3 | this study |
| YCH4Sc | 0.267 | kg/kg | Methane yield coefficient for carbohydrates | Calculated by stochiometric coefficients |
| YCH4Sp | 0.375 | kg/kg | Methane yield coefficient for proteins | Calculated by stochiometric coefficients |
| YCH4Sl | 0.707 | kg/kg | Methane yield coefficient for lipids | Calculated by stochiometric coefficients |
| YCH4Sh | 0.284 | kg/kg | Methane yield coefficient for hemicelluloses | Calculated by stochiometric coefficients |
| YCH4Slg | 0.535 | kg/kg | Methane yield coefficient for lignin | Calculated by stochiometric coefficients |

The hydrolysis constant and death rate parameters were fitted to CO2 production profile from Komilis and Ham (2006).

Yield coefficients in heterotrophic growth reactions microorganisms

| n | Yn(X/S) | Yn (X/O2) | Yn (X/NH3) | Yn (X/CO2) | Yn (X/H2O) |
| --- | --- | --- | --- | --- | --- |
| 13 | 0.35 | 0.61286157 | 0.70588235 | 0.44571751 | 0.71648551 |
| 14 | 0.35 | 0.34845815 | 2.49062545 | 0.29323448 | 1.79198847 |
| 15 | 0.35 | 0.15679879 | 6.64705882 | 0.16528408 | 0.38083635 |
| 16 | 0.35 | 0.61286157 | 0.70588235 | 0.44571751 | 0.71648551 |
| 17 | 0.35 | 0.34845815 | 2.49062545 | 0.29323448 | 1.79198847 |
| 18 | 0.35 | 0.15679879 | 6.64705882 | 0.16528408 | 0.38083635 |
| 19 | 0.35 | 0.61286157 | 0.70588235 | 0.44571751 | 0.71648551 |
| 20 | 0.35 | 0.34845815 | 2.49062545 | 0.29323448 | 1.79198847 |
| 21 | 0.35 | 0.15679879 | 6.64705882 | 0.16528408 | 0.38083635 |
| 22 | 0.35 | 0.19196328 | 6.64705882 | 0.13960966 | 0.3294401 |
| 23 | 0.35 | 0.61286157 | 0.70588235 | 0.44571751 | 0.71648551 |
| 24 | 0.35 | 0.34845815 | 2.49062545 | 0.29323448 | 1.79198847 |
| 25 | 0.35 | 0.15679879 | 6.64705882 | 0.16528408 | 0.38083635 |
| 26 | 0.35 | 0.19196328 | 6.64705882 | 0.13960966 | 0.3294401 |
| 27 | 0.35 | 0.59266453 | 14.5294118 | 0.41509283 | 0.83045149 |
| 28 | 0.35 | 0.34183472 | 2.06986995 | 0.25426471 | 2.72849867 |
| 29 | 0.35 | 0.1487208 | 14.5294118 | 0.16088253 | 0.41080206 |
| 30 | 0.35 | 0.18993587 | 14.5294118 | 0.13645628 | 0.35162785 |
| 31 | 0.35 | 0.21008868 | 14.5294118 | 0.19653097 | 0.62593621 |
| 32 | 0.35 | 0.59266453 | 14.5294118 | 0.41509283 | 0.83045149 |
| 33 | 0.35 | 0.34183472 | 2.06986995 | 0.25426471 | 2.72849867 |
| 34 | 0.35 | 0.1487208 | 14.5294118 | 0.16088253 | 0.41080206 |
| 35 | 0.35 | 0.18993587 | 14.5294118 | 0.13645628 | 0.35162785 |
| 36 | 0.35 | 0.21008868 | 14.5294118 | 0.19653097 | 0.62593621 |

*Calculated by stochiometric coefficients from 6

**X is the biomass involved in the reaction

# Details of the parametrization

| Parameters | Possible choices |
| --- | --- |
| Waste composition | - Fraction of food waste |
| Mass ratio of biowaste/structuring material | - Possible structuring material : wood chips: - 3/1 - 8/1 |
| Technology | - Home composting - Industrial composting: - Forced aeration : 0.3 l/mn.kg - Passive aeration : 0.08 l/mn.kg - Turning heaps : 0.08 l/mn.kg |
| Pre-treatment technology | - Drum-screen + Shredder + Piston press - Shredder + Piston press + Screw press - Separation-mill - Pulp + Drum-screen |
| Ambient temperature in composting site | - 20°C - 5°C |
| Temperature in field | - 0-5°C - 5.1-10°C - 10.1-15°C - 15.1-20°C |
| Infiltration rate of soil | - Low - Medium - High |
| Soil pH | - Acidic - Neutral - Basic |
| Precipitation levels | - 0-2 mm - 2-5 mm - 5-10 mm - >10 mm |

Energy requirement of composting technology

| Equipment | Energy consumption | Unit | Type of energy | Reference |
| --- | --- | --- | --- | --- |
| Sorting technology |  |  |  |  |
| Drum-screen | 0.04 | kWh/t | Electricity | 14 |
| Shredder | 4.14 | kWh/t | Electricity | 14 |
| Piston press | 5.8 | kWh/t | Electricity | 14 |
| Screw press | 2.55 | kWh/t | Electricity | 14 |
| Separation-mill | 9 | kWh/t | Electricity | 15 |
| Pulp | 22 | kWh/t | Electricity | 14 |
| Aeration |  |  |  |  |
| Compost fan | 9 | kWh/t | Electricity | 16 |
| Turning heaps machine | 1.02 | l/t | Diesel | 17 |

# Initial values for the simulation

| Variables | A (kg/kg TM) | B (kg/ kg TM) |
| --- | --- | --- |
| Carbohydrate | 0.1642 | 0.1709 |
| Protein | 0.0392 | 0.0466 |
| Lipid | 0.0452 | 0.0358 |
| Hemicellulose | 0.0142 | 0.0142 |
| Cellulose | 0.0347 | 0.0360 |
| Lignin | 0.0315 | 0.0318 |
| Inert | 0.0127 | 0.029 |

# Inventory data for LCA simulations

| Processus | Flow type | Flow | Quantity | Unit | Observation |
| --- | --- | --- | --- | --- | --- |
| Collection | Input | Biowaste | 1 | kg |  |
|  |  | Transport | 3.00E-02 | tkm | Distance of waste collection is 30 km |
|  | Output | Collected biowaste | 1 | kg |  |
| Pretreatment | Input | Collected biowaste | 1 | kg |  |
|  |  | Electricity | 0.01 | kWh | Technology used : Drum-screen + Shredder + Piston press |
|  |  | Incineration of reject | 0.23 | kg |  |
|  | Output | Sorted biowaste | 0.77 | kg |  |
| Correction of C/N | Input | Sorted biowaste | 0.77 | kg |  |
|  |  | Wood chips | 9.63E-2 | kg |  |
|  | Output | Mixed biowaste | 0.87 | kg |  |
| Active composting | Input | Mixed biowaste | 0.87 | kg |  |
|  |  | Electricity | 0.01 | kWh | Only for active composting |
|  | Output | Treated biowaste | 1 | kg |  |
|  |  | Distance of compost transport | 20 | km | Transport of compost in tkm depends on the mass of compost produced, wich is predicted by the model for each scenario |

# Validation of active composting model

| Author | Technology | Feedstock | Duration | CO2 (kg.kgTM-1) | CH4 (kg.kgTM-1) | N2O (kg.kgTM-1) | NH3 (kg.kgTM-1) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 18 | Experimental HC | Biowaste | One year | 0.177 – 0.252 | 0.4e-3 – 4.2e-3 | 0.30e-3 – 0.45e-3 | - |
| 19 | Experimental HC | Food waste + pruning waste | 96 days |  | 0.3e-3 |  | 0.025e-3 |
| 20 | Home composting | Biowaste + green waste | 12 and 51 weeks | 0.139 – 0.215 | 0.788e-3 – 2.185e-3 | 0.192e-3 – 0.454e-3 | 0.474e-3 – 0.972e-3 |
|  | Windrow composting | Biowaste | 9 and 12 and 21 weeks | 0.049 – 0.115 | 0.243e-3 – 0.293e-3 | 0.027e-3 – 0.116e-3 | 0.052e-3 – 0.576e-3 |
| 21 | Home composting | OFMSW + pruning waste | 12 weeks |  | 0.115e-3 | 0.492e-3 | 0.613e-3 |
| 22 | Four industrial compostings (unaerated and aerated) | Biowaste + greenwaste | 3 to 14 weeks |  | 0.154e-3 – 13.030e-3 | 0 – 0.788e-3 | 0.053e-3 – 0.163e-3 |
|  | A | Biowaste + wood chips | 90 days | 0.3133 | 1.4746e-4 | 0.06484e-3 | 0.142e-3 |
|  | B | Biowaste + wood chips | 90 days | 0.3005 | 1.4702e-4 | 0.06483e-3 | 0.202e-3 |
|  | B_5 | Biowaste + wood chips | 90 days | 0.3091 | 1/4743e-4 | 0.050e-3 | 0.217e-3 |
|  | B_passive | Biowaste + wood chips | 90 days | 0.2949 | 2.1728e-4 | 0.0678e-3 | 0.055e-3 |

# LCA results

- Contribution of processus

Total ecosystem quality

|  | A | B | B_5°C | B_passive |
| --- | --- | --- | --- | --- |
| Transport of biowaste | 2.46E-11 | 2.45599E-11 | 2.45599E-11 | 2.45599E-11 |
| Electricity consumption | 3.78E-11 | 3.78085E-11 | 3.78085E-11 | 1.85282E-11 |
| Wood chips consumption | 4.20E-10 | 4.20222E-10 | 4.20222E-10 | 4.20222E-10 |
| CH4direct | 1.29E-11 | 1.28694E-11 | 1.29045E-11 | 1.90193E-11 |
| NH3direct | 5.12E-11 | 7.26751E-11 | 7.8152E-11 | 1.9948E-11 |
| N2Odirect | 4.69E-11 | 4.68629E-11 | 3.61173E-11 | 4.89876E-11 |
| Substitution of peat | -2.47E-11 | -2.59263E-11 | -2.51688E-11 | -2.63901E-11 |
| Substitution of Nfertilizer | -4.35E-11 | -5.94969E-11 | -5.83071E-11 | -6.17275E-11 |
| Substitution of Pfertilizer | -4.04E-12 | -5.74478E-12 | -5.74478E-12 | -5.74478E-12 |
| Substitution of Kfertilizer | -1.06E-11 | -1.74947E-11 | -1.74947E-11 | -1.74947E-11 |
| Transport of compost | 1.45E-11 | 1.4467E-11 | 1.44899E-11 | 1.44832E-11 |
| N2Onet from use-on-land | 4.56E-11 | 4.86197E-11 | 4.9069E-11 | 4.8725E-11 |
| NO3net from use-on-land | 6.73E-13 | 7.51315E-13 | 7.53813E-13 | 7.58311E-13 |
| NH3net from use on-land | 5.74E-11 | 7.36291E-11 | 7.12294E-11 | 7.61192E-11 |
| (PO4)3-net from use-on-land | -6.43E-12 | -9.13216E-12 | -9.13216E-12 | -9.13216E-12 |
| On-land CO2 emission from replacement of peat | -6.18E-10 | -6.48547E-10 | -6.296E-10 | -6.60149E-10 |
| Incineration of sorting reject | 5.70E-11 | 5.70157E-11 | 5.70157E-11 | 5.70157E-11 |
| Total | 6.09E-11 | 4.31E-11 | 5.69E-11 | -3.23E-11 |

Total human health

|  | A | B | B_5°C | B_passive |
| --- | --- | --- | --- | --- |
| Transport of biowaste | 1.06E-08 | 1.06E-08 | 1.06E-08 | 1.06E-08 |
| Electricity consumption | 1.91E-08 | 1.91E-08 | 1.91E-08 | 9.38E-09 |
| Wood chips consumption | 8.86E-09 | 8.86E-09 | 8.86E-09 | 8.86E-09 |
| CH4direct | 4.04E-09 | 4.03E-09 | 4.04E-09 | 5.95E-09 |
| NH3direct | 1.85E-08 | 2.62E-08 | 2.82E-08 | 7.19E-09 |
| N2Odirect | 1.61E-08 | 1.61E-08 | 1.24E-08 | 1.68E-08 |
| Substitution of peat | -9.07E-09 | -9.51E-09 | -9.23E-09 | -9.68E-09 |
| Substitution of Nfertilizer | -1.86E-08 | -2.54E-08 | -2.49E-08 | -2.64E-08 |
| Substitution of Pfertilizer | -2.36E-09 | -3.36E-09 | -3.36E-09 | -3.36E-09 |
| Substitution of Kfertilizer | -4.88E-09 | -8.08E-09 | -8.08E-09 | -8.08E-09 |
| Transport of compost | 6.17E-09 | 6.16E-09 | 6.17E-09 | 6.17E-09 |
| N2Onet from use-on-land | 1.56E-08 | 1.67E-08 | 1.68E-08 | 1.67E-08 |
| NO3net from use-on-land | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 |
| NH3net from use on-land | 2.07E-08 | 2.65E-08 | 2.57E-08 | 2.74E-08 |
| (PO4)3-net from use-on-land | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 |
| On-land CO2 emission from replacement of peat | -2.05E-07 | -2.15E-07 | -2.09E-07 | -2.19E-07 |
| Incineration of sorting reject | 4.15E-08 | 4.15E-08 | 4.15E-08 | 4.15E-08 |
| Total | -7.88E-08 | -8.56E-08 | -8.09E-08 | -1.16E-07 |

Total natural resources

|  | A | B | B_5°C | B_passive |
| --- | --- | --- | --- | --- |
| Transport of biowaste | 8.00E-04 | 8.00E-04 | 8.00E-04 | 8.00E-04 |
| Electricity consumption | 4.30E-04 | 4.30E-04 | 4.30E-04 | 2.11E-04 |
| Wood chips consumption | 5.57E-04 | 5.57E-04 | 5.57E-04 | 5.57E-04 |
| CH4direct | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 |
| NH3direct | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 |
| N2Odirect | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 |
| Substitution of peat | -2.48E-04 | -2.60E-04 | -2.52E-04 | -2.65E-04 |
| Substitution of Nfertilizer | -1.05E-03 | -1.44E-03 | -1.41E-03 | -1.49E-03 |
| Substitution of Pfertilizer | -4.34E-05 | -6.17E-05 | -6.17E-05 | -6.17E-05 |
| Substitution of Kfertilizer | -2.73E-04 | -4.52E-04 | -4.52E-04 | -4.52E-04 |
| Transport of compost | 4.81E-04 | 4.80E-04 | 4.81E-04 | 4.81E-04 |
| N2Onet from use-on-land | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 |
| NO3net from use-on-land | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 |
| NH3net from use on-land | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 |
| (PO4)3-net from use-on-land | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 |
| On-land CO2 emission from replacement of peat | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 |
| Incineration of sorting reject | 5.02E-04 | 5.02E-04 | 5.02E-04 | 5.02E-04 |
| Total | 1.15E-03 | 5.58E-04 | 5.95E-04 | 2.81E-04 |

- Contribution of mid-point impact

Total ecosystem quality

|  | A | B | B_5°C | B_passive |
| --- | --- | --- | --- | --- |
| acidification: terrestrial | 1.14502E-10 | 1.47679E-10 | 1.51038E-10 | 9.38818E-11 |
| climate change: freshwater ecosystems | -1.30286E-14 | -1.41956E-14 | -1.39256E-14 | -1.46182E-14 |
| climate change: terrestrial ecosystems | -4.76896E-10 | -5.19618E-10 | -5.09727E-10 | -5.35069E-10 |
| ecotoxicity: freshwater | 1.35872E-12 | 1.1447E-12 | 1.1571E-12 | 8.00148E-13 |
| ecotoxicity: marine | 2.82113E-13 | 2.38963E-13 | 2.41456E-13 | 1.72417E-13 |
| ecotoxicity: terrestrial | 1.95743E-12 | 1.51406E-12 | 1.53867E-12 | 1.2222E-12 |
| eutrophication: freshwater | 5.12075E-12 | 1.70233E-12 | 1.73687E-12 | -6.78752E-13 |
| eutrophication: marine | 6.77543E-13 | 7.54584E-13 | 7.57143E-13 | 7.61038E-13 |
| land use | 4.01256E-10 | 3.99759E-10 | 3.99896E-10 | 3.98568E-10 |
| photochemical oxidant formation: terrestrial ecosystems | 1.11469E-11 | 9.61938E-12 | 9.73321E-12 | 8.55837E-12 |
| water use: aquatic ecosystems | 6.95246E-17 | 1.60793E-17 | 2.31847E-17 | -2.12069E-17 |
| ecosystem quality - water use: terrestrial ecosystems | 1.55394E-12 | 3.59389E-13 | 5.18201E-13 | -4.73996E-13 |

Total human health

|  | A | B | B_5°C | B_passive |
| --- | --- | --- | --- | --- |
| climate change: human health | -1.5789E-07 | -1.72035E-07 | -1.68802E-07 | -1.7726E-07 |
| human toxicity: carcinogenic | 8.98331E-09 | 7.88973E-09 | 7.96372E-09 | 6.90637E-09 |
| human toxicity: non-carcinogenic | 1.79369E-08 | 1.67176E-08 | 1.67863E-08 | 1.49879E-08 |
| ionising radiation | 2.63463E-11 | 2.38063E-11 | 2.421E-11 | 4.9298E-12 |
| ozone depletion | 7.03075E-10 | 7.16794E-10 | 6.45239E-10 | 7.30499E-10 |
| particulate matter formation | 5.11542E-08 | 6.10074E-08 | 6.2343E-08 | 3.89488E-08 |
| photochemical oxidant formation: human health | 7.59624E-11 | 6.58381E-11 | 6.65984E-11 | 5.86239E-11 |
| water use: human health | 2.55537E-10 | 5.90996E-11 | 8.52153E-11 | -7.7946E-11 |

Total natural resources

|  | A | B | B_5°C | B_passive |
| --- | --- | --- | --- | --- |
| energy resources: non-renewable, fossil | 0.001087417 | 0.000514703 | 0.000550386 | 0.0002468 |
| material resources: metals/minerals | 6.69722E-05 | 4.33389E-05 | 4.4759E-05 | 3.37551E-05 |

References

(1)	Ge, J.; Huang, G.; Huang, J.; Zeng, J.; Han, L. Particle-Scale Modeling of Methane Emission during Pig Manure/Wheat Straw Aerobic Composting. Environ. Sci. Technol. 2016, 50 (8), 4374–4383. https://doi.org/10.1021/acs.est.5b04141.

(2)	Arah, J. R. M.; Stephen, K. D. A Model of the Processes Leading to Methane Emission from Peatland. Atmospheric Environment 1998, 32 (19), 3257–3264. https://doi.org/10.1016/S1352-2310(98)00052-1.

(3)	US EPA. Process Design Manual for Nitrogen Control. https://nepis.epa.gov (accessed 2023-09-25).

(4)	de Guardia, A.; Petiot, C.; Benoist, J. C.; Druilhe, C. Characterization and Modelling of the Heat Transfers in a Pilot-Scale Reactor during Composting under Forced Aeration. Waste Management 2012, 32 (6), 1091–1105. https://doi.org/10.1016/j.wasman.2011.12.028.

(5)	Lin, Y. P.; Huang, G. H.; Lu, H. W.; He, L. Modeling of Substrate Degradation and Oxygen Consumption in Waste Composting Processes. Waste Management 2008, 28 (8), 1375–1385. https://doi.org/10.1016/j.wasman.2007.09.016.

(6)	Sole-Mauri, F.; Illa, J.; Magrí, A.; Prenafeta-Boldú, F. X.; Flotats, X. An Integrated Biochemical and Physical Model for the Composting Process. Bioresource Technology 2007, 98 (17), 3278–3293. https://doi.org/10.1016/j.biortech.2006.07.012.

(7)	Didier, O. Modélisation de la stabilisation de la matière organique et des émissions gazeuses au cours du compostage d’effluents d’élevage. phdthesis, INSA de Toulouse, 2013. https://tel.archives-ouvertes.fr/tel-00935691 (accessed 2022-11-16).

(8)	Sander, R. Henry’s Law Constants. https://henrys-law.org/henry/casrn/7664-41-7 (accessed 2025-04-18).

(9)	Lin, Y.-M.; Tay, J.-H.; Liu, Y.; Hung, Y.-T. Biological Nitrification and Denitrification Processes. In Biological Treatment Processes; Handbook of environmental engineering; 2009; Vol. 8.

(10)	Wilshusen, J. H.; Hettiaratchi, J. P. A.; Stein, V. B. Long-Term Behavior of Passively Aerated Compost Methanotrophic Biofilter Columns. Waste Management 2004, 24 (7), 643–653. https://doi.org/10.1016/j.wasman.2003.12.006.

(11)	Watson, A.; Stephen, K.; Nedwell, D.; Arah, J. Oxidation of Methane in Peat: Kinetics of CH4 and O2 Removal and the Role of Plant Roots. Soil Biology and Biochemistry 1997, 29 (8), 1257–1267. https://doi.org/10.1016/S0038-0717(97)00016-3.

(12)	Hanson, J. L.; Yeşiller, N.; Onnen, M. T.; Liu, W.-L.; Oettle, N. K.; Marinos, J. A. Development of Numerical Model for Predicting Heat Generation and Temperatures in MSW Landfills. Waste Management 2013, 33 (10), 1993–2000. https://doi.org/10.1016/j.wasman.2013.04.003.

(13)	The Engineering ToolBox. https://www.engineeringtoolbox.com/air-specific-heat-capacity-d_705.html.

(14)	Beaufort, R.; Lacout, A. Inventaire et Performances Des Technologies de Déconditionnement Des Biodéchets, 2016.

(15)	RECYC-QUEBEC. Technologies de Désemballage Des Produits Alimentaires Périmés; 2015.

(16)	ECS STAFF. Compost Fans: How Much Energy Do they Consume?. Engineered Compost Systems. https://compostsystems.com/compost-fans-energy/.

(17)	Ravary-Berger, F. Développement d’un Outil Paramétré Pour l’optimisation Environnementale de La Séquence de Traitement de Matières Résiduelles Avec Prise En Compte de La Qualité Des Flux de Matières, 2021.

(18)	Andersen, J. K.; Boldrin, A.; Christensen, T. H.; Scheutz, C. Mass Balances and Life Cycle Inventory of Home Composting of Organic Waste. Waste Management 2011, 31 (9), 1934–1942. https://doi.org/10.1016/j.wasman.2011.05.004.

(19)	Colón, J.; Martínez-Blanco, J.; Gabarrell, X.; Artola, A.; Sánchez, A.; Rieradevall, J.; Font, X. Environmental Assessment of Home Composting. Resources, Conservation and Recycling 2010, 54 (11), 893–904. https://doi.org/10.1016/j.resconrec.2010.01.008.

(20)	Amlinger, F.; Peyr, S.; Cuhls, C. Green House Gas Emissions from Composting and Mechanical Biological Treatment. Waste Manag Res 2008, 26 (1), 47–60. https://doi.org/10.1177/0734242X07088432.

(21)	Martínez-Blanco, J.; Colón, J.; Gabarrell, X.; Font, X.; Sánchez, A.; Artola, A.; Rieradevall, J. The Use of Life Cycle Assessment for the Comparison of Biowaste Composting at Home and Full Scale. Waste Management 2010, 30 (6), 983–994. https://doi.org/10.1016/j.wasman.2010.02.023.

(22)	Matlach, J.; Knoll, L.; Reinelt, T.; Cuhls, C.; Stinner, W.; Daniel-Gromke, J. Greenhouse Gas Emissions from Anaerobic and Aerobic Treatment of Biowaste and Green Waste. Waste Management 2025, 207, 115097. https://doi.org/10.1016/j.wasman.2025.115097.